Deep Learning Based Multi-Object Robotic Grasp Detection using Segment Anything Model

# P. SRI NIDHI

Department of Mechanical Engineering Amrita Vishwa Vidyapeetham Bengaluru, Karnataka, India

# INDHUSREE REDDY M

Department of Mechanical Engineering Amrita Vishwa Vidyapeetham Bengaluru, Karnataka, India

bl.en.u4rae23031@bl.students.amrita.edu bl.en.u4rae23064@bl.students.amrita.edu

Abstract— Robotic grasp detection plays an important role in modern robotics and automation systems because robots must identify stable grasp positions before performing object manipulation tasks. Traditional Computer Vision based methods mainly depend on handcrafted features such as edge detection, contour extraction, thresholding, and geometric analysis. These approaches often fail in cluttered and occluded environments due to overlapping objects, varying object shapes, noisy backgrounds, and illumination changes. In this work, a Deep Learning based multi-object robotic grasp detection system is proposed using Convolutional Neural Networks (CNNs) and the Segment Anything Model (SAM). A clutter grasp dataset containing RGB images and grasp annotations is used for training the CNN model. The proposed pipeline performs image preprocessing, grasp annotation extraction, K-Means clustering, object-wise segmentation, CNN-based grasp centre prediction, and grasp rectangle generation. SAM is used for accurate segmentation of individual objects from cluttered scenes, while the CNN predicts stable grasp centre coordinates for robotic gripping. The proposed approach successfully detects grasp locations for multiple objects in cluttered environments and improves robustness compared to traditional Computer Vision methods. Experimental results demonstrate that the hybrid CNN and SAM based framework effectively performs multi-object grasp localization suitable for robotic manipulation applications.

Index Terms — Robotic Grasp Detection, Deep Learning, CNN, Segment Anything Model, Multi-Object Grasping, Robotics, Cluttered Scenes.

- Introduction

Robotic grasp detection is a fundamental task in robotics and autonomous manipulation systems because robots must determine stable grasp positions before interacting with objects. Robotic grasping is widely used in industrial automation, warehouse robotics, medical robotics, service robots, and autonomous robotic systems. Accurate grasp prediction improves robotic efficiency, reduces grasp failures, and enables safe object handling in real-world applications.

Traditional Computer Vision based robotic grasp detection techniques mainly rely on handcrafted image features such as contours, edges, corner points, thresholding, and geometric descriptors. Although these approaches perform reasonably well in controlled environments, they fail in cluttered scenes where multiple objects overlap each other. Occlusions, lighting variations, noisy backgrounds, and object shape diversity further reduce the effectiveness of traditional methods.

Recent advancements in Deep Learning have significantly improved robotic perception and grasp detection. Convolutional Neural Networks (CNNs) automatically learn spatial and visual features directly from image data, making grasp prediction more robust and accurate. Deep Learning models reduce the dependency on handcrafted features and improve generalization in complex environments.

In this project, a hybrid Deep Learning and segmentation-based framework is proposed for multi-object robotic grasp detection. A CNN model is trained using grasp annotations to predict grasp centre coordinates, while the Segment Anything Model (SAM) is used for accurate object segmentation in cluttered environments. The proposed system performs preprocessing, segmentation, object-wise cropping, grasp prediction, and grasp rectangle generation to produce stable robotic grasp outputs.

The major contribution of this work is the integration of SAM-based segmentation with CNN-based grasp center prediction for robust multi-object grasp localization in cluttered scenes.

- Literature Survey

- Deep Learning for Robotics Grasp Detection

Lenz et al. [1] proposed one of the earliest Deep Learning based robotic grasp detection frameworks using hierarchical feature learning techniques. Their work introduced a two-stage Deep Learning architecture for predicting robotic grasp rectangles from RGB-D images. The proposed method demonstrated that neural networks could automatically learn grasp-related visual features instead of relying on handcrafted descriptors. Their research significantly improved robotic grasp localization accuracy compared to traditional Computer Vision methods.

Kumra and Kanan [2] developed a robotic grasp detection system using Deep Convolutional Neural Networks (CNNs). Their approach utilized transfer learning with pre-trained CNN architectures for robotic grasp prediction. The proposed framework improved feature extraction capability and demonstrated higher grasp localization accuracy in cluttered scenes. Their work highlighted the effectiveness of CNN-based feature learning for robotic manipulation tasks.

Levine et al. [3] proposed an end-to-end Deep Learning framework for robotic grasping using large-scale robotic interaction data. Their system learned grasping policies directly from robot trial-and-error experiences. The proposed method significantly improved grasp robustness and demonstrated that Deep Learning can effectively learn robotic manipulation strategies from data.

- Real time Robotic Grasping Approaches

Morrison et al. [4] introduced the Generative Grasping Convolutional Neural Network (GG-CNN) for real-time robotic grasp detection. Their architecture directly predicted grasp quality, grasp angle, and gripper width from depth images. The GG-CNN model achieved fast inference speed suitable for real-time robotic applications and improved robotic grasp synthesis performance.

Mahler et al. [5] proposed the Dex-Net framework for robust robotic grasp planning using synthetic grasp datasets and analytic grasp metrics. Their work combined physics-based grasp analysis and Deep Learning for improving grasp stability and reliability. Dex-Net became one of the most widely used robotic grasp datasets and frameworks for robotic grasp learning research.

- Object Detection and Feature Learning

Morrison et al. [4] introduced the Generative Grasping Convolutional Neural Network (GG-CNN) for real-time robotic grasp detection. Their architecture directly predicted grasp quality, grasp angle, and gripper width from depth images. The GG-CNN model achieved fast inference speed suitable for real-time robotic applications and improved robotic grasp synthesis performance.

Mahler et al. [5] proposed the Dex-Net framework for robust robotic grasp planning using synthetic grasp datasets and analytic grasp metrics. Their work combined physics-based grasp analysis and Deep Learning for improving grasp stability and reliability. Dex-Net became one of the most widely used robotic grasp datasets and frameworks for robotic grasp learning research.

- Segmentation Techniques for Robotics Vision

AKirillov et al. [8] proposed the Segment Anything Model (SAM), a generalized image segmentation framework capable of accurately segmenting both known and unseen objects. SAM demonstrated strong generalization capability in cluttered environments and improved segmentation robustness significantly. The segmentation capability of SAM makes it highly suitable for robotic manipulation applications involving multi-object cluttered scenes.

Traditional segmentation techniques mainly relied on contour extraction, thresholding, edge detection, and morphological operations. However, these approaches often failed under object occlusions and noisy environments. SAM-based segmentation overcomes many of these limitations by generating accurate segmentation masks using foundation-model-based feature learning.

- Research Objectives

The proposed study consists of several interconnected objectives aimed at developing a robust Deep Learning based robotic grasp detection framework for cluttered and occluded environments. The objectives are designed to improve robotic grasp localization, object-wise segmentation, and multi-object grasp prediction using Deep Learning and segmentation techniques.

To begin with, the first objective of this work is to develop a robust Convolutional Neural Network (CNN) model for robotic grasp center prediction. The CNN must learn spatial and visual features directly from RGB image data and predict stable grasp center coordinates accurately. Special attention is given to proper train-test separation, image preprocessing, normalization, and reduction of overfitting during training. The model should generalize effectively for different object shapes and cluttered scenes without relying on handcrafted Computer Vision features.

Second, the proposed work aims to improve multi-object grasp localization in cluttered environments using the Segment Anything Model (SAM). The segmentation framework should accurately separate individual objects even under overlapping and occluded conditions. By generating object-wise segmentation masks and bounding regions, the proposed system reduces background interference and improves grasp localization performance for robotic manipulation tasks.

Third, the system aims to integrate K-Means clustering for grasp annotation grouping and object-wise grasp region separation. Since cluttered scenes contain multiple grasp annotations corresponding to different objects, clustering is required to group nearby grasp points effectively. The clustering process should improve object-wise grasp center estimation and enable more stable robotic grasp generation in multi-object environments.

Fourth, the proposed framework aims to generate stable grasp rectangles suitable for robotic gripping applications. The CNN predicts grasp center coordinates while geometric processing is used to generate grasp rectangles corresponding to two-finger robotic grippers. The objective is to improve robotic grasp stability and grasp localization robustness for real-world robotic manipulation tasks.

Finally, the overall performance of the proposed system must be evaluated using multiple experimental criteria including grasp localization capability, segmentation quality, clutter handling robustness, CNN convergence behaviour, and generalization performance. The study also considers implementation-related factors such as preprocessing efficiency, segmentation complexity, and inference performance for robotic grasp detection applications.

- Methdology

- Input Image Acquisition

The first stage of the proposed system involves acquiring RGB images containing multiple objects arranged in cluttered environments. The dataset images consist of overlapping objects with different shapes, orientations, and background conditions. These cluttered scenes are used to train and evaluate the robustness of the robotic grasp detection framework. Into a number of groups of medical information.

- Image Pre-processing

Before training the CNN model, several preprocessing operations are performed on the dataset images. The RGB images are loaded using OpenCV and resized to 224 × 224 pixels to ensure fixed-size CNN input. Image normalization is then performed by scaling pixel values between 0 and 1. Normalization improves training stability and enables faster convergence of the neural network during optimization.

The preprocessing stage also removes invalid images and ensures consistent input formatting for the CNN architecture.

- Grasp Annotation Extraction

Each dataset image contains a corresponding annotation text file that stores grasp-related information. The annotation file contains grasp center coordinates, grasp angle, and gripper width values in the format:

y x angle width

where:

- y represents the vertical grasp coordinate,

- x represents the horizontal grasp coordinate,

- angle represents the gripper orientation angle,

- width represents the gripper opening width.

These annotations are extracted and processed to generate training targets for the CNN model.

- K-Means Clustering for Grasp Region Grouping

Since cluttered scenes contain multiple grasp annotations corresponding to different objects, K-Means clustering is used to group nearby grasp points. The clustering process separates grasp regions belonging to different objects and improves object-wise grasp localization.

The clustering algorithm identifies cluster centres for grasp annotations and computes representative grasp locations for each object. This step improves the robustness of grasp prediction in cluttered multi-object scenes.

- Co-ordinate Scaling

After clustering, the grasp coordinates are scaled according to resized image dimensions. Since the original image size differs from the CNN input size, coordinate scaling ensures correct mapping between grasp annotations and resized images.

Scaled grasp coordinates are then used as training targets for CNN-based grasp centre prediction.

- Segment Anything Model (SAM) Segmentation

The Segment Anything Model (SAM) is integrated into the proposed system for accurate object-wise segmentation. SAM generates segmentation masks for individual objects present in cluttered scenes. The segmentation process improves object separation and reduces background interference during grasp prediction.

The segmentation masks are filtered based on object area and bounding box information to identify valid object regions. SAM provides strong generalization capability and effectively segments both known and unseen objects.

- Object-wise Cropping

After segmentation, each detected object is cropped individually using bounding boxes generated from segmentation masks. Object-wise cropping isolates relevant object regions and eliminates unnecessary background information.

This process improves CNN prediction accuracy because the network focuses only on the segmented object instead of the complete cluttered scene.

- CNN-Based Grasp Center Prediction

The cropped object images are passed into the Convolutional Neural Network (CNN) for grasp prediction. The CNN architecture extracts spatial and visual features from the segmented object images and predicts stable grasp centre coordinates.

The CNN predicts only the grasp centre coordinates (x, y), while grasp rectangles are generated using geometric processing techniques. ReLU activation functions are used in hidden layers, and a linear activation function is used in the output layer because grasp prediction is treated as a regression problem.

- Grasp Rectangle Detection

Using the predicted grasp centers, geometric processing is performed to generate grasp rectangles suitable for two-finger robotic grippers. The grasp rectangle represents the estimated gripping region for stable robotic manipulation.

The generated grasp rectangles are visualized on segmented objects for multi-object grasp localization analysis.

- Final Multi-Object Grasp Detection

In the final stage, the proposed system produces stable grasp predictions for multiple segmented objects present in cluttered scenes. The hybrid integration of SAM segmentation and CNN-based grasp prediction improves robustness, object separation, and grasp localization accuracy.

The final output consists of:

- segmented object regions,

- object-wise grasp centres,

- grasp rectangles,

- multi-object grasp localization visualization.

The proposed framework demonstrates improved performance in cluttered robotic manipulation environments compared to traditional Computer Vision based grasp detection approaches

- Results and Discussion

- CNN Training Performance

The CNN model was successfully trained using grasp annotation data extracted from the clutter grasp dataset. During training, the model learned important spatial and visual features required for robotic grasp centre prediction. The training and validation loss gradually decreased over multiple epochs, indicating stable convergence of the network.

The use of image normalization, dropout layers, and proper train-test separation improved training stability and reduced overfitting. The CNN effectively generalized for different object shapes and clutter conditions present in the dataset.

The final trained model successfully predicted grasp centre coordinates (x, y) for segmented objects and produced stable grasp localization outputs.

- Performance of K Means Clustering

K-Means clustering effectively grouped nearby grasp annotations corresponding to different objects in cluttered scenes. The clustering process improved object-wise grasp region separation and reduced confusion between overlapping grasp points.

By computing representative cluster centers for grasp annotations, the system generated more stable and meaningful grasp targets for CNN training. The clustering approach also improved grasp localization robustness in multi-object environments.

- SAM Based Object Segmentation Results

K The Segment Anything Model (SAM) demonstrated strong segmentation capability for cluttered object scenes. SAM accurately separated individual objects even under partial occlusions and overlapping conditions. The segmentation masks generated by SAM improved object-wise cropping and significantly reduced background interference during grasp prediction.

The filtering of segmentation masks based on object area further improved segmentation quality by removing extremely small and irrelevant regions. The segmented object outputs demonstrated that SAM can effectively generalize for different object categories without requiring class-specific training.

- Multi-Object Grasp Detection Results

The proposed framework successfully generated grasp predictions for multiple objects simultaneously. After segmentation and object-wise cropping, the CNN predicted stable grasp centre coordinates for each segmented object individually.

Grasp rectangles generated using predicted grasp centers visually represented stable gripping locations suitable for two-finger robotic grippers. The final outputs demonstrated that the proposed system can handle cluttered scenes containing multiple overlapping objects while maintaining stable grasp localization performance.

The generated grasp outputs showed improved object separation and grasp stability compared to traditional Computer Vision based grasp detection methods.

Fig. X. Final grasp localization results generated using CNN-based grasp prediction.

- Overall System Performance and Discussion

The overall performance of the proposed robotic grasp detection framework demonstrates that the integration of Convolutional Neural Networks (CNNs) and Segment Anything Model (SAM) significantly improves grasp localization capability in cluttered and occluded environments. The CNN model successfully learned spatial and visual image features required for grasp center prediction and showed stable convergence during training. The use of preprocessing operations such as normalization, coordinate scaling, and K-Means clustering improved training consistency and object-wise grasp region separation. K-Means clustering effectively grouped nearby grasp annotations corresponding to different objects and reduced confusion between overlapping grasp points in cluttered scenes.

The Segment Anything Model (SAM) demonstrated strong segmentation capability by accurately separating multiple objects even under overlapping and partially occluded conditions. Object-wise cropping reduced unnecessary background interference and improved grasp prediction robustness for individual objects. The final system successfully generated stable grasp rectangles suitable for two-finger robotic grippers and produced effective multi-object grasp localization outputs. Compared to traditional Computer Vision based methods, the proposed Deep Learning and segmentation-based framework demonstrated improved robustness, clutter handling capability, and generalization performance, making it suitable for future robotic manipulation and autonomous grasping applications.

Limitations and Future Directions

- Model and Computational Challenges

Although the proposed framework demonstrates effective robotic grasp localization in cluttered environments, several model-related challenges still exist in the current implementation. The CNN architecture predicts only the grasp center coordinates (x, y) and does not directly estimate complete grasp pose parameters such as grasp orientation angle and gripper width. Because of this limitation, additional geometric processing is required to generate grasp rectangles suitable for robotic gripping applications.

The proposed system also relies heavily on the Segment Anything Model (SAM) for object-wise segmentation. While SAM provides accurate segmentation performance, it significantly increases computational complexity and inference time. Processing cluttered high-resolution images using SAM requires large memory resources and high computational power, making real-time deployment difficult on low-resource robotic platforms.

Another challenge is that the proposed framework uses only RGB image information for grasp prediction. Depth information from RGB-D sensors is not incorporated into the current system. The absence of depth information may reduce grasp estimation accuracy for objects with complex 3D geometries, varying surface heights, and irregular object orientations.

Furthermore, the system performance depends strongly on segmentation quality. Incorrect segmentation masks, partial object boundaries, or inaccurate object cropping may negatively affect CNN-based grasp prediction accuracy. Extremely dense cluttered scenes containing severe object overlap may also reduce segmentation robustness and grasp localization performance.

- Practical and Implementation Constraints

The dataset used in this work contains approximately 505 cluttered RGB images, which is relatively small compared to large-scale Deep Learning datasets. Limited dataset diversity may affect the generalization capability of the CNN model for highly complex real-world robotic environments containing different object textures, lighting conditions, and unseen object categories.

The current implementation mainly focuses on software-based robotic grasp prediction and visualization. Physical robotic arm integration and real-time robotic grasp execution are not implemented in the proposed framework. Therefore, the system has not been experimentally validated on actual robotic hardware platforms.

In addition, the proposed framework performs supervised grasp prediction using labelled grasp annotations but does not include reinforcement learning or feedback-based robotic optimization. As a result, the robot cannot improve grasping performance dynamically through interaction with the environment.

The proposed system also requires multiple preprocessing operations such as image normalization, grasp annotation extraction, K-Means clustering, segmentation filtering, and object-wise cropping before grasp prediction. These additional processing stages increase implementation complexity and overall system latency.

Finally, the proposed framework has not been evaluated under highly dynamic environments involving moving objects, changing lighting conditions, or real-time industrial robotic manipulation tasks. Therefore, further optimization and real-world testing are required before deployment in practical robotic applications.

- Conclusion

In this work, a Deep Learning based multi-object robotic grasp detection framework was successfully developed using Convolutional Neural Networks (CNNs) and the Segment Anything Model (SAM). The proposed system combined image preprocessing, grasp annotation extraction, K-Means clustering, object-wise segmentation, CNN-based grasp centre prediction, and grasp rectangle generation for robotic grasp localization in cluttered environments.

The CNN model effectively learned spatial image features and predicted stable grasp centre coordinates from RGB images, while SAM improved object-wise segmentation and reduced background interference in cluttered scenes. The integration of segmentation and Deep Learning techniques significantly improved grasp localization robustness compared to traditional Computer Vision based methods.

Experimental results demonstrated that the proposed framework successfully generated grasp predictions for multiple objects simultaneously and handled overlapping object conditions effectively. The system also showed improved generalization capability for different object arrangements and cluttered environments.

Although the current implementation predicts only grasp center coordinates and does not include real-time robotic arm execution, the proposed framework provides a strong foundation for intelligent robotic manipulation systems. Future improvements such as depth-based grasp estimation, full grasp pose prediction, lightweight segmentation models, and real-time robotic integration can further enhance system performance and practical applicability in industrial and autonomous robotic applications.

References

- [1] I. Lenz, H. Lee, and A. Saxena, “Deep Learning for Detecting Robotic Grasps,” The International Journal of Robotics Research, vol. 34, no. 4–5, pp. 705–724, 2015. DOI: 10.1177/0278364914549607.

- [2] S. Kumra and C. Kanan, “Robotic Grasp Detection using Deep Convolutional Neural Networks,” in Proceedings of IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), 2017. DOI: 10.1109/IROS.2017.8202237.

- [3] S. Levine, P. Pastor, A. Krizhevsky, and D. Quillen, “Learning Hand-Eye Coordination for Robotic Grasping with Deep Learning and Large-Scale Data Collection,” The International Journal of Robotics Research, vol. 37, no. 4–5, pp. 421–436, 2018. DOI: 10.1177/0278364917710318.

- [4] D. Morrison, P. Corke, and J. Leitner, “Closing the Loop for Robotic Grasping: A Real-time, Generative Grasp Synthesis Approach,” Robotics: Science and Systems (RSS), 2018. DOI: 10.15607/RSS.2018.XIV.003.

- [5] J. Mahler et al., “Dex-Net 2.0: Deep Learning to Plan Robust Grasps with Synthetic Point Clouds and Analytic Grasp Metrics,” Robotics: Science and Systems (RSS), 2017. DOI: 10.15607/RSS.2017.XIII.058.

- [6] J. Redmon, S. Divvala, R. Girshick, and A. Farhadi, “You Only Look Once: Unified Real-Time Object Detection,” in Proceedings of IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2016. DOI: 10.1109/CVPR.2016.91.

- [7] K. He, X. Zhang, S. Ren, and J. Sun, “Deep Residual Learning for Image Recognition,” in Proceedings of IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2016. DOI: 10.1109/CVPR.2016.90.

- [8] A. Kirillov et al., “Segment Anything,” in Proceedings of IEEE International Conference on Computer Vision (ICCV), 2023. DOI: 10.1109/ICCV51070.2023.00371.